[image: image1.jpg]FLEXIBLE FLYER®

Yerf-Dog

Utility Vehicles / Go-Karts / ATVs




Time Required

Average of 30 Minutes per unit

Required Tools

1.  Rubber Mallet

2.  Flat-Head Screwdriver

3.  Retainer-ring Pliers

4.  12mm Open-end Wrench

1. 17mm Boxed-end Wrench

Procedure:

1. Inspect the serial number on the kart and manufacture dates.  If the serial numbers or manufacture dates indicate that this unit will need the required repairs, then continue to step 2. 

· If the kart is made between July 20, 2004 and March 04, 2005 All four of the retaining rings and the ball joints must be replaced.

Models Effected:  3206, 4209, 42092, 42093, 42101

[image: image2.jpg]The Serial Label Decal YERF-DUG

can be _fOUﬂ.d on th_e DESC: FULL SUSP. KART
carton in this location MODEL» 4209
* SERIAL# 136246

a9 located in this area of

e carton indicates the go-kart has
- /been repaired or was produced after
the designated dates




[image: image3.jpg]The Serial Label
Decal can be found
on the Go-Kart in
these locations




Go-Karts manufactured 

BETWEEN DATES JULY 20, 2004 and MARCH 04, 2005

 must be repaired.

2. Remove nut attaching the A-Arm Ball Joints to the spindle.  Complete this for all (4) ball joints. (See Images below) This will allow the wheel / spindle assembly to be removed from the kart.  There will be a left and right wheel assembly.

[image: image4.jpg]Spindle

Ball Joint



[image: image5.jpg]V.

12mm
Open-end Wrench

17mm
Boxed-end We




3. Using a Screw Driver remove the retaining ring from around the top of the ball joint and discard. 
 Complete this process for all (4) ball joints & Retainer rings.
NOTE:  The retainer ring shown in these images is the new style ring.  The old style ring will be a rounded style wire ring that will have to be removed by placing a screwdriver and pushing it off.  
4. Remove the ball joints (ALL 4 Ball Joints and Rings) by tapping lightly on top of the ball joint with the rubber mallet from the a-arms and discard.

[image: image6.jpg]Spindle



[image: image7.jpg]/ »
Spindle




Note:  Do not mix the old ball joints and the new ball joints. As soon as you remove the ball joint from the kart destroy the ball joint by deforming the ball joint housing (silver part).  These ball joints must never be used again.

5. Replace the old ball joints with new ball joints.  This process may require you to hammer them into place.  Make sure not to damage the ball joint stud or threads during this process.  This may require lightly tapping on the edge of the ball joint.

6. Secure the new ball joints in place using the provided new retainer rings.  These rings can only be put into place using the snap-ring tool.  Once the ring is in place make sure that it is seated properly into the upper groove on the ball joint.

[image: image8.jpg]Ball Joint

Retainer Ring




7. Inspect all hardware to insure that it has been secured in place and there are no loose components.

Technical Illustrations

[image: image9.jpg]REMOVE BALL JOINT FROM SPINDLE

ASSEMBLY BY REMOVING THE NUT

FROM THE BALL JOINT.

REMOVE EACH RETAINING RING

FROM THE TOP SIDE OF THE BALL JOINT

USING A PICK TO GET UNDER THE RING

A TRANSMISSION RING REMOVAL TOOL WORKS WELL

REMOVE BALL JOINT FROM A-ARM BY LIGHTLY
TAPPING ON THE TOP OF THE BALL JOINT

DISCARD THE OLD BALL JOINTS AND RETAINING RING

SPINDLE ASSEMBLY

-A-ARM HOLE

1. Place new ball joint through hole at en
of A-Arm and secure with snap ring.

AARM N Spindle

2. Place ball joint through hole in spindle-
and secure with nut.

Bottom ball joint shown already assembled.

STEP 2




Questions

If for any reason the store location demands a return, have them contact

 Yerf Dog Customer service @ 1 (800) 926-6554.













